LOCKDOWN— Running Instructions

Objective.  Quality control of cross-dating or measurement of ring-width series in a “.rwl”file.

Notation.  The following notation is used for MATLAB© interaction.

[…..] = menu box, requiring you to choose from one of several options

{….} =  edit window, requiring you to enter input data

<…>=file window, requiring you to click on a file

>>  = MATLAB© command prompt

Preliminary steps.  Have ready:  1) the unzipped Lockdown function (m-files) in the current MATLAB© working directory or a directory that is on the MATLAB© path, 2) a “.rwl” file with ring-width measurements, in the current working directory, 3) <optional> an ITRDB-format “.crn” file, if that is what is to used for a master.

Outline format.  These steps for running Lockdown are in the form of a bulleted outline that approximately follows the flow of Lockdown.

Strategy.  Typically, the starting point is a “.rwl” file of ring widths that have been cross-dated visually or with skeleton plots. If you decide to use a “.crn” file for the master, you must have that file available in the working directory.  The other approach is to iteratively build up a reliable master from the test series in the .rwl file.  The first time you run Lockdown in this iterative mode, each series is tested against an “all others” master built from all series except the test series.  This initial master will generally be contaminated by some bad series.  In this initial run you can specify which series you want to be in a revised master.  The ids of series in this master are stored in “ids_master_initial.mat”.   Lockdown is then run again, but calling for the master to built from series with ids from “ids_master_initial.mat”.  Additional well-dated series can be added to a revised master, whose ids are saved in a new file,  “ids_master.mat”.  From then on the procedure is iterative, each time adding to the series in “ids_master.mat”.

The choice of width of the sliding window is very important to the analysis.  The default is 50 years, based on the assumption that 50 years is a reasonable length of series to identify cross-dating.  This assumption may be good for some types (e.g., species, geographical regions, site-types) and not for others.   With some tree-ring data, 50 years may be too long for the available sample sizes.  For example, some riparian data sets may have many trees less than 50 years old.  You can lower the window width at one of the menus to deal with such data, but it is a good idea in building the master to begin with as wide a window as possible.  Subsequent runs can then use narrower windows, allowing shorter series to be evaluated and added to the master if justified by the correlations and plots.

There are two other modes, not yet programmed completely, for running Lockdown: 1) running an undated test series against a master, and 2) simple time plots of tentatively dated short series in the rwl file with the master.  In (1), a floating ring-width series assigned years 1,2,…  (undated) is checked against the master or a .crn to find a “best” match.  The intended application is to check for where a piece of old remnant wood might date.  In (2), simple time plots are produced of high-passed series if ring widths and the master.  This mode is intended for getting plots for samples that may be too short for quantitative statistical checking with correlation analysis.  For example, a test series may overlap the master by only 5 years.  It may still be worthwhile to see that there is no radical miss-match in the pattern for those 5 years.  If you ask to run either of the two modes describe in this paragraph, Lockdown will report a “fatal error” and bomb out.

- >>Lockdown…..  at the matlab prompt, this starts the program.  
[Choose Mode]:  “Test tentatively dated series”
[Choose form of master]: “Scaled high-pass average of all other core indices”

- {Program Settings} :  Here you can change the defaults of spline wavelength (20 yr), width of sliding window (50 yr) , and offset (5 yr).  Lockdown computes correlation of each series with a “master” in a sliding window.  The series are high-pass filtered by taking the difference of the original series and the smoothed series – smoothed by the spline.   When satisfied, “OK”

- <rwl infile with test series>:  double-click (or single click + “open”) on the rwl file with ring width series to be tested.  Here you may get a message box that some series have less than a recommended (e.g. 52 yr) overlap with master (the “master” at this point is a combination of all other series).  The message box will list the ids of cores that cannot be tested. Lockdown will still run, but you will want to know that some series cannot be test.  If you get that message, press “OK” to get rid of the message and move on.

- [Choose]:  You can choose one of three operations.  1) Check a test series, 2) View summary stats, 3) Modify makeup of master.  First choice should be to “View summary stats”

Fig 1 pops up with the median, minimum and maximum correlation for each core.  You may want to tug at the window corners to stretch the window out if too compresses.  The cores are numbered sequentially beginning with 1, as they were ordered in the rwl file.  The blue circle gives the median of all correlations ( 50-year periods, offset by 5 yr) between the test series and a master composed of all other series.  Likewise, the red x’s give the maximum and minimum of those correlations.  You will be suspicious of series with low medians, but particularly of series with high medians but very low minimum correlations.  Those would have one or more 50-year periods of unusually low correlation with the master.  Say you have identified such a series.  The next thing is to find out which one it is.

- Now choose “Check a test series” to get a menu list of all series, with sequential number, core id and time coverage.  Click on the entry for the suspect series and up comes a new figure window (Fig 2) and new menu.

  - Fig 2-top shows the times series of sliding correlations of the test series with the master (solid blue) and with the master offset by + 1 year or – 1 year (red and green thin lines).  The x-axis is time, the ending year of the 50-year window.  This plot lets you identify any periods the correlation with the master breaks down.   The title at the top of Fig 2 includes information about how much of the test series is “checkable”.  This depends on how much available overlap there is with the master.  The longest series in the rwl will not be checkable over its full length.

  - Fig2-middle is a time series of standard deviations of the master and test series.  These plots are sometimes useful for explaining low correlation, as series are unlikely to maintain high correlation if they do not vary (low standard deviation in their 50-year window).

  - Fig  2-bottom shows the full time series of test series and master.

Say from Fig 2-top you identify a 50-year period with a bad correlation.  Now you will want to view a close-up of the test and master for that segment.  You will do this with the existing menu.

- [Choose ] Choose “Click on a point …”  , move the cross-hair over the desired part of the plots in Fig 2-top, and up will pop the close-up in Fig 2-bottom.  You can keep going back and forth to the menu here and choose other points in Fig 2-top to see how the series line up in any given 50 yrs of the test series.  The zoomed plots in Fig 2-bottom immediately bring to the eye dating offsets and possible measurement errors.    In some cases, the series are perfectly synchronous up to a certain year, and obviously offset after that, and possibly synchronous again later on.    When you are satisified, you can choose “No zoom – move on” from the menu, and go back to the upper level menu that lets you check other series.

- [Choose]  “Modify makeup of master” …. to begin assigning cores to an initial master.  A menu of all test cores will then appear. Cores in this menu are numbered sequentially by their position in the “.rwl” file.  An entry in the menu includes 1) sequence number, 2) core id, 3) time coverage, and 4) “Y” or “N”.   This “Y” or “N” refers to the series being in the master already or not.  Initially, all series are marked with “N” because a master is yet to be built.  

Click on the box for a series to add that series to master.  “N” will change to “Y”.  As a guide, bring up Figure window 1 on the computer screen beside this menu of series.  Click on a set of series with high correlations, keeping in mind you also want reasonable time coverage in the master.  After the set of desired series has been marked, click “Accept”.

- [ “Update ids_master_initial.mat with revised set of master cores?” ].  Click on “Yes”.

- [“Close all windows?].  Click on “Yes” or “No”.   “No” keeps the windows in case you want to copy them to some document.  Either “Yes” or “No” will close the program.

Next iteration.  Running through the steps above has produced a .mat file name ids_master_initial.mat in your working directory.  Those ids define a “master” that includes strong series only.  Now each series not in this master can be tested against the master.

You now repeat steps 1-9, and only a few items described below, are different.

-In step (1), at [Choose form of master], pick  “Scaled high-pass average of core indices whose ids will be read in”

-After step (3), you are prompted for “Infile with ids of cores for building master”.  Click on “ids_master_initial.mat.  A message comes up telling you which cores may not be testable because of insufficient overlap with the master.  This message is for your information.  Click on OK to move on.  But note that at the previous step you could narrow the window (default is 50 years), which would lead to more cores being checkable.

-In item (7), when you decide to “modify makeup of master”, you will see that the series currently in the master have “Y” beside their names in the menu.  You can now add to those.

-In item (8), when you save the ids for the revised master, you will be saving them to a file ids_master.mat, rather than to ids_master_initial.mat.

Subsequent iterations.  The above sequence is repeated, with the only difference that now you click on “ids_master.mat” as the input file with ids of the master.  That file is modified each time time you add series.  The idea is that you identify problem series, go back to look at the wood sample and correct mistakes, changing the “.rwl” file, run Lockdown again to see if all series are OK, and so on.

Multiple chronologies.  Beware that Lockdown will overwrite the file ids_master.mat if you run Lockdown on a new “.rwl” file in the same working directory as a site you have just finished.  For that reason, once you finish with a site, rename the file ids_master.mat so that the name is specific to a site name.  For example, “ids_masterTUM.mat”.